100071011 Computer Networks 2023-2024-2

Project-1

Reliable File Transfer using Go-Back-N protocol

Specification

| 学号 (Student ID) | 1820221053 |
| --- | --- |
| 姓名 (Name) | 曾泇睷 |
| 班号 (Class No.) | 07812201 |
| 授课教师 (Instructor) | 胡琳梅 教师 |

School of Computer

Beijing Institute of Technology

April 12, 2024

- Requirement Analysis

The project aims to implement a reliable file transfer system over UDP using the Go-Back-N (GBN) protocol. The system must ensure reliable data transfer despite potential packet losses and errors, simulating real-world network conditions.

Key requirements include:

• Reliable Transfer: Use GBN to handle packet losses and errors, ensuring the receiver gets an exact copy of the sender’s file.

• UDP Sockets: Implement communication using UDP, which is unreliable, requiring GBN for reliability.

• Full-Duplex Communication: Both hosts act as sender and receiver simul-tenuously, transferring a 3 MB file.

• Configuration File: Read parameters (e.g., ports, timeout, error/loss rates) from a configuration file.

• Logging: Record detailed communication states for each PDU sent/received in a specified format.

• Performance Analysis: Compute metrics like throughput, retransmissions, and duration.

Main Issues include:

• Managing the sliding window mechanism in GBN.

• Handling timeouts and retransmissions effectively.

• Ensuring data integrity through checksums.

• Synchronizing sender and receiver in full-duplex mode.

- Design

  - Frame Structure

The Protocol Data Unit (PDU) structure includes:

• type: 0 (data), 1 (ACK), 2 (last data).

• seqNum: 32-bit sequence number for data or ACK number.

• checksum: CRC-CCITT for error detection.

• data: Payload, up to 4KB.

  - Checksum Generation and Checking

The CRC-CCITT algorithm is employed to generate and verify checksums, ensuring data integrity.

  - Sliding Windows

The sender uses a configurable window size N, while the receiver expects PDUs in sequence, discarding out-of-order packets as per GBN.

  - Buffering

The sender buffers PDUs within the window, and the receiver may buffer out-of-order PDUs, though GBN typically discards them.

  - Sequence Spaces and Numbers

Sequence numbers are 32-bit integers, providing a large sequence space to avoid wraparound issues.

  - Acknowledgment Rules

Cumulative ACKs are used, where an ACK for sequence number k confirms receipt of all PDUs up to k.

  - Retransmissions

Upon timeout, the sender retransmits all PDUs in the window starting from the oldest unacknowledged PDU.

  - Timeout

Timer A single timer tracks the oldest unacknowledged PDU.

  - UDP Socket

UDP sockets facilitate PDU transmission and reception.

  - System Model

Each host runs two threads: one for sending and one for receiving, enabling full-duplex communication.

  - Functions/Classes

• PDU struct: Defines the PDU structure.

• Logger class: Handles asynchronous logging.

• sender and receiver functions: Implement core logic.

  - Flow Charts, State Diagrams, and Time-Sequence Diagrams

• Sender Flow: Send PDUs within window, process ACKs, handle timeouts.

• Receiver Flow: Receive PDUs, verify checksums, send ACKs.

• States: Sender (Idle, Sending, Waiting for ACK); Receiver (Waiting for PDU, Sending ACK).

• Time-Sequence: Normal operation and error scenarios (e.g., PDU loss).

  - Error Simulations

Random PDU errors and losses are simulated based on configurable rates.

  - Logging

Asynchronous logging minimizes performance overhead.

  - Configuration File

Parameters include ports, data size, window size, timeout, error/loss rates, and file names.

  - Multi-Threading

Separate threads manage sending and receiving tasks.

  - Initialization

The system reads parameters from a configuration file during startup.

- Development and Implementation

  - Development Tools

• OS: Windows

• Language: C++

• Libraries: Winsock2

  - Project Structure

• pdu.h: PDU structure and functions.

• utils.h: Configuration and logging utilities.

• sender.cpp: Sender implementation.

• receiver.cpp: Receiver implementation.

• main.cpp: Program entry point.

  - Critical Functions

• calculateCRC: Computes CRC-CCITT checksum.

• serializePDU / deserializePDU: PDU serialization and deserialization.

• Logger class: Manages asynchronous logging.

  - Sample Code

The following code snippets highlight key functionalities of the system:

    - PDU Serialization

This function converts a PDU structure into a byte stream for UDP transmission, handling both data and ACK PDUs.

std::vector<char> serializePDU(const PDU& pdu, size_t dataSize) {

size_t totalSize = (pdu.type == TYPE_ACK) ? 7 : (7 + dataSize);

std::vector<char> buffer(totalSize);

buffer[0] = pdu.type;

memcpy(&buffer[1], &pdu.seqNum, sizeof(pdu.seqNum));

memcpy(&buffer[5], &pdu.checksum, sizeof(pdu.checksum));

if (pdu.type == TYPE_DATA || pdu.type == TYPE_LAST_DATA) {

memcpy(&buffer[7], pdu.data, dataSize);

}

return buffer;

}

    - Checksum Calculation

The CRC-CCITT algorithm ensures data integrity by generating a checksum for each PDU.

uint16_t calculateCRC(const char* data, size_t length) {

const uint16_t POLYNOMIAL = 0x1021;

uint16_t crc = 0xFFFF;

for (size_t i = 0; i < length; ++i) {

crc ^= (static_cast<uint16_t>(data[i]) << 8);

for (int j = 0; j < 8; ++j) {

if (crc & 0x8000) {

crc = (crc << 1) ^ POLYNOMIAL;

} else {

crc <<= 1;

}

}

}

return crc;

}

    - Sender Logic (Window Management)

This excerpt from the sender function illustrates the sliding window mechanism, including PDU transmission and timeout handling

while (nextSeqNum < base + config.windowSize && nextSeqNum < stats.totalPDUs) {

PDU& pdu = pdus[nextSeqNum];

bool shouldSend = dis(gen) >= config.lossRate;

if (shouldSend) {

PDU sendPdu = pdu;

if (dis(gen) < config.errorRate) {

sendPdu.data[0] ^= 0x01; // Flip a bit to simulate error

}

std::vector<char> buffer = serializePDU(sendPdu, config.dataSize);

sendto(sock, buffer.data(), buffer.size(), 0, (sockaddr*)&remoteAddr, sizeof(remoteAddr));

std::string status = (nextSeqNum >= base) ? "New" : "TO";

stats.totalSends++;

if (status == "TO") stats.retransmitCount++;

Logger logger("transfer.log");

logger.log(std::to_string(sendCount++) + ", pdu_to_send=" + std::to_string(nextSeqNum) + ", status=" + status + ", ackedNo=" + std::to_string(base));

}

if (base == nextSeqNum) {

sendTimes[nextSeqNum] = std::chrono::steady_clock::now();

timerRunning = true;

}

nextSeqNum++;

}

    - Asynchronous Logging

The Logger class reduces file I/O overhead by buffering log messages and writing them in batches.

class Logger {

public:

Logger(const std::string& logFileName) : logFile(logFileName, std::ios::app), running(true) {

if (!logFile) {

throw std::runtime_error("Cannot open log file");

}

loggingThread = std::thread(&Logger::loggingFunction, this);

}

~Logger() {

stop();

}

void log(const std::string& message) {

{

std::lock_guard<std::mutex> lock(queueMutex);

logQueue.push(message);

}

cv.notify_one();

}

void stop() {

{

std::lock_guard<std::mutex> lock(queueMutex);

running = false;

}

cv.notify_one();

if (loggingThread.joinable()) {

loggingThread.join();

}

}

private:

std::ofstream logFile;

std::queue<std::string> logQueue;

std::mutex queueMutex;

std::condition_variable cv;

std::thread loggingThread;

bool running;

void loggingFunction() {

while (running || !logQueue.empty()) {

std::vector<std::string> batch;

{

std::unique_lock<std::mutex> lock(queueMutex);

cv.wait(lock, [this]{ return !logQueue.empty() || !running; });

while (!logQueue.empty()) {

batch.push_back(logQueue.front());

logQueue.pop();

}

}

for (const auto& msg : batch) {

logFile << msg << std::endl;

}

logFile.flush(); // Ensure logs are written immediately

}

}

};

- System Deployment, Startup, and Use

  - Deployment

Compile the code using Visual Studio or the cl compiler. Prepare configuration files for each host.

  - Startup

Run the executable with a configuration file, e.g., FileTransfer.exe config1.txt.

  - Use

The system automatically initiates file sending and receiving based on the configuration.

- System Test

  - Unit Test

• Tested PDU serialization and deserialization.

• Verified checksum calculation and validation.

  - Integrated Test

• Tested file transfer with varying error/loss rates.

• Confirmed received files match originals.

  - Test Cases

• Normal transfer without errors.

• Transfer with simulated errors.

• Transfer with simulated losses.

  - Screenshots

- Performance and Analysis

  - Performance Metrics

• Total PDUs sent.

• Total send attempts.

• Number of timeouts.

• Number of retransmissions.

• Total transfer time.

  - Analysis

Performance was evaluated by varying data size, window size, error/loss rates, and timeout values. Metrics were plotted to assess their impact.

  - Data Sheet

| Config | Data Size | Window Size | Error Rate | Loss Rate | Time (s) |
| --- | --- | --- | --- | --- | --- |
| 1 | 3 MB | 5 | 0.03 | 0.03 | 903.19 |
| 2 | 3MB | 10 | 0.05 | 0.05 | 206.66 |

Table 1: Performance metrics under different configurations.

- Summary or Conclusions

This project provided hands-on experience with the GBN protocol, resulting in a reliable file transfer system. Challenges included managing sliding windows, handling timeouts, and ensuring data integrity with checksums. The technical solution involved UDP sockets, multi-threading, and CRC-CCITT checksums. The system successfully achieved reliable transfers despite simulated errors and losses, with key features such as full-duplex communication, configurable parameters, and detailed logging.

- References

[1] Computer networks / Andrew S. Tanenbaum, David J. Wetherall. -- 5th ed.

[2] UDP Communication. https://wiki.python.org/moin/UdpCommunication

[3] An Intro to Threading in Python . https://realpython.com/intro-to-python-threading/

[4] recv and recvfrom, socket programming using python. https://stackoverflow.com/questions/36115971/recv-and-recvfrom-socket-programming-using-python

[5] Go-Back-N Protocol. https://www.baeldung.com/cs/networking-go-back-n-protocol

- Comments

This project proved to be an intensive learning journey that challenged me both intellectually and physically. Initially overwhelmed by the complexity of implementing network protocols from scratch, I dedicated two consecutive sleepless nights to research, coding, and iterative debugging. Through systematic study of technical documentation and persistent experimentation, I gradually conquered each technical hurdle. While the process demanded extraordinary effort, it profoundly deepened my comprehension of computer network architectures and significantly enhanced my software engineering capabilities. The hands-on experience with protocol design and error simulation proved invaluable, transforming theoretical textbook concepts into practical engineering wisdom.